CMPG 325
COMPUTER NETWORKS

MILESTONE 1

Network Requirements and Preliminary Design

Phomolong Insurance Brokers (Vryburg)

Lonwabo Cwele

| Student Name | Lonwabo Cwele |
| --- | --- |
| Student Number | 45225672 |
| Project Number | CMPG325-2026-012 |
| Assigned Network | 172.30.0.0/23 |
| Submission | Milestone 1 |

North-West University
2026

# Table of Contents

1. Introduction ............................................. 3

2. Client Profile and Design Assumptions ............................................. 3

3. Network Requirements ............................................. 4

4. Departmental and User Requirements ............................................. 5

5. Proposed VLAN Structure ............................................. 6

6. Network Devices and Services ............................................. 7

7. VoIP and Traffic Requirements ............................................. 8

8. Physical Building and Floor Layout ................................... 9

9. Scalability and Future Expansion ..................................... 10

10. Addressing and Routing Strategy ...................................... 10

11. Preliminary VLSM Addressing Plan ................................... 11

12. Design Summary ................................................ 12

# 1. Introduction

This document presents the preliminary network requirements and design assumptions for Phomolong Insurance Brokers in Vryburg. The purpose of Milestone 1 is to establish a realistic, scalable and manageable network foundation before implementation in Cisco Packet Tracer. The design uses the assigned IPv4 address block 172.30.0.0/23 and is structured to support departmental separation, inter-VLAN communication, VoIP, shared network services, guest access and future organisational growth.

# 2. Client Profile and Design Assumptions

Where the project brief does not prescribe exact quantities, realistic estimates are used. Phomolong Insurance Brokers is therefore modelled as a small-to-medium insurance brokerage operating from two currently occupied floors, with an additional floor expected to become available in a future financial year.

| Planning Item | Assumption |
| --- | --- |
| Current employees | Approximately 50 |
| Current occupied floors | 2 |
| Future expansion | 1 additional floor |
| Business functions | Management, Finance, Claims, Brokers/Sales, Administration & HR, IT Support, Reception/Customer Service |
| Employee endpoints | Approximately 50 PCs/laptops |
| VoIP requirement | Up to 50 IP phones |
| Guest wireless users | Up to 20 concurrent devices |
| Growth allowance | Approximately 25–30% for existing departments |

# 3. Network Requirements

The network must be designed and simulated in Cisco Packet Tracer.

The assigned address block 172.30.0.0/23 must be used efficiently.

Departments must be logically separated using VLANs.

Inter-VLAN routing must be implemented using Router-on-a-Stick and router subinterfaces.

The network must support the planned adoption of VoIP handsets through a dedicated voice VLAN.

Internal servers must be placed in a dedicated server VLAN.

Guest wireless users must be logically separated from internal departmental users.

Network infrastructure must include suitable managed switches, a router, access points, servers, printers and end devices.

The design must allow future expansion without requiring a complete addressing redesign.

The final implementation must support connectivity testing, troubleshooting and clear technical documentation.

# 4. Departmental and User Requirements

| Department | Estimated Current Users | Primary Function |
| --- | --- | --- |
| Management | 5 | Executive and supervisory users |
| Finance | 8 | Financial records, payments and accounting |
| Claims | 10 | Claims processing and customer case management |
| Brokers / Sales | 12 | Insurance sales, quotations and client communication |
| Administration & HR | 7 | Administrative and staff-related functions |
| IT Support | 4 | Network and technical administration |
| Reception / Customer Service | 4 | Front-office and customer assistance |

Total estimated current employees: 50.

# 5. Proposed VLAN Structure

| VLAN ID | VLAN Name | Purpose |
| --- | --- | --- |
| 10 | MANAGEMENT | Management workstations |
| 20 | FINANCE | Finance department |
| 30 | CLAIMS | Claims department |
| 40 | SALES | Brokers and sales |
| 50 | ADMIN_HR | Administration and Human Resources |
| 60 | IT | IT support |
| 70 | RECEPTION | Reception and customer service |
| 80 | VOICE | VoIP handsets |
| 90 | SERVERS | Internal servers |
| 100 | GUEST | Guest wireless devices |
| 110 | NET_MGMT | Network infrastructure management |
| 120 | FUTURE | Reserved future-floor capacity |

# 6. Network Devices and Services

| Device | Estimated Quantity | Role |
| --- | --- | --- |
| Router | 1 | Router-on-a-Stick inter-VLAN routing |
| Managed switches | 3 | Departmental access, VLANs and trunk links |
| Employee PCs/laptops | ~50 | User endpoints |
| IP phones | Up to 50 | VoIP communication |
| Servers | 4 | DHCP/DNS, file, intranet/web and backup/application services |
| Network printers | 5 | Departmental network printing: Claims, Admin/HR, Sales, Finance and Management |
| Wireless access points | ~3 | Staff/guest wireless coverage |

## 6.1 Departmental Printer Allocation

Five network printers are included in the physical design so that the Packet Tracer topology remains consistent with the stated requirements. Each printer will be connected to the relevant access switch and placed in the same data VLAN as the department it serves.

| Printer Name | Department / VLAN | Access Switch |
| --- | --- | --- |
| PRN-CLAIMS | Claims / VLAN 30 | SW2-GROUND |
| PRN-ADMIN | Admin & HR / VLAN 50 | SW2-GROUND |
| PRN-SALES | Sales / VLAN 40 | SW3-FIRST |
| PRN-FINANCE | Finance / VLAN 20 | SW3-FIRST |
| PRN-MGMT | Management / VLAN 10 | SW3-FIRST |

# 7. VoIP and Traffic Requirements

Management intends to adopt VoIP handsets. Voice traffic will therefore be separated from ordinary data traffic using VLAN 80. A workstation may connect through an IP phone while the phone and workstation remain in different logical VLANs. Voice is delay-sensitive, so the design recognises latency, jitter and packet loss as important performance considerations.

For planning purposes, an active VoIP call is estimated at approximately 80–100 kbps including protocol overhead. If 20 calls are active simultaneously, approximately 1.6–2.0 Mbps of traffic may be generated. Ordinary office traffic is expected to be moderate and will include email, web access, insurance applications, file transfers, printing, cloud services and internal server access.

# 8. Physical Building and Floor Layout

The network is designed around two currently occupied floors, with central network services located in a dedicated server/network room. A future second floor is reserved for expansion. Each occupied floor is served by a managed access switch that connects back to the core switch using trunk links. This arrangement keeps the physical design easy to manage while supporting VLANs across the building.

| Location | Departments / Devices | Primary Switch | Design Role |
| --- | --- | --- | --- |
| Server / Network Room | R1 router, SW1-CORE, four servers and central network equipment | SW1-CORE | Central routing, services and distribution |
| Ground Floor | Reception/Customer Service, Claims, Administration & HR, Guest Wi-Fi, PRN-CLAIMS, PRN-ADMIN and representative VoIP devices | SW2-GROUND | Ground-floor user access |
| First Floor | Management, Finance, Brokers/Sales, IT Support, PRN-MGMT, PRN-FINANCE, PRN-SALES and representative VoIP devices | SW3-FIRST | First-floor user access |
| Future Second Floor | Future staff, additional APs, phones, printers and other endpoints as required | Future access switch | Reserved expansion using VLAN 120 and planned address capacity |

The future second floor is included in the logical design but is not treated as currently deployed equipment. VLAN 120 and subnet 172.30.0.64/26 are reserved for this expansion. When the floor becomes operational, an additional managed switch can be connected to the core/distribution layer through a trunk link and the reserved VLAN can be extended to the new floor.

# 9. Scalability and Future Expansion

The design provides approximately 25–30% growth capacity for existing departments and reserves a dedicated future VLAN for the proposed additional floor. Additional unused address space in the assigned /23 block is also retained. This allows new staff, devices and services to be added without restructuring the entire addressing scheme.

# 10. Addressing and Routing Strategy

Variable Length Subnet Masking (VLSM) will be used so that subnet sizes reflect the actual number of hosts required by each VLAN. Larger networks, such as Voice and Future Expansion, receive larger subnets while smaller departments receive smaller subnets. The first usable address in each subnet is reserved as the default gateway. These gateway addresses will later be configured on router subinterfaces as part of the Router-on-a-Stick implementation.

# 11. Preliminary VLSM Addressing Plan

| VLAN | Name | Network | Gateway | Usable Host Range | Broadcast |
| --- | --- | --- | --- | --- | --- |
| 80 | VOICE | 172.30.0.0/26 | 172.30.0.1 | 172.30.0.1–172.30.0.62 | 172.30.0.63 |
| 120 | FUTURE | 172.30.0.64/26 | 172.30.0.65 | 172.30.0.65–172.30.0.126 | 172.30.0.127 |
| 100 | GUEST | 172.30.0.128/27 | 172.30.0.129 | 172.30.0.129–172.30.0.158 | 172.30.0.159 |
| 40 | SALES | 172.30.0.160/27 | 172.30.0.161 | 172.30.0.161–172.30.0.190 | 172.30.0.191 |
| 30 | CLAIMS | 172.30.0.192/27 | 172.30.0.193 | 172.30.0.193–172.30.0.222 | 172.30.0.223 |
| 20 | FINANCE | 172.30.0.224/28 | 172.30.0.225 | 172.30.0.225–172.30.0.238 | 172.30.0.239 |
| 50 | ADMIN_HR | 172.30.0.240/28 | 172.30.0.241 | 172.30.0.241–172.30.0.254 | 172.30.0.255 |
| 10 | MANAGEMENT | 172.30.1.0/28 | 172.30.1.1 | 172.30.1.1–172.30.1.14 | 172.30.1.15 |
| 60 | IT | 172.30.1.16/29 | 172.30.1.17 | 172.30.1.17–172.30.1.22 | 172.30.1.23 |
| 70 | RECEPTION | 172.30.1.24/29 | 172.30.1.25 | 172.30.1.25–172.30.1.30 | 172.30.1.31 |
| 90 | SERVERS | 172.30.1.32/29 | 172.30.1.33 | 172.30.1.33–172.30.1.38 | 172.30.1.39 |
| 110 | NET_MGMT | 172.30.1.40/29 | 172.30.1.41 | 172.30.1.41–172.30.1.46 | 172.30.1.47 |

Unallocated capacity remains from 172.30.1.48 to 172.30.1.255, providing further room for additional services, devices or future redesign requirements.

# 12. Design Summary

The preliminary design provides a structured foundation for the Phomolong Insurance Brokers network. The physical design separates the current Ground Floor and First Floor through dedicated managed access switches connected to a central core switch in the server/network room. Departmental VLANs improve logical organisation and security, Router-on-a-Stick will provide inter-VLAN routing, a dedicated voice VLAN accommodates the VoIP change request, and VLSM uses the allocated 172.30.0.0/23 block efficiently. Five departmental network printers are included in their respective departmental VLANs. The future second floor is explicitly reserved through VLAN 120 and planned address capacity, while further unused address space remains available for growth. The next stage is to construct the physical and logical topology in Cisco Packet Tracer and begin device configuration.